ECO Process

Revision History

| Rev | Description of Change | Revision Date |
| --- | --- | --- |
| A | Initial Release | 09/20/02 |
| B | Added process clarification to section 6 regarding ECO steps, and when it is appropriate to use each, added acronyms | 11/07/02 |
| C | Raised the Quality Standard for ECO releases- Revised Scope and Policy; added Inspection Checkpoints to ECO process, revised stakeholders roles & responsibilities and moved to Appx A; added definitions; added Process Governance; Created Appx B | 06/25/03 |
| D | Multiple clerical updates including updates to the ECO Steps (breaking Step 6 to 6A, 6B, 6C and Step 7 to 7A and 7B) throughout the document, adding references to the Customer Change Notification in the Reference and GPM sections, adding a listing of changes that can be made on ECOs (Section 6.2.3), and adding D&DC Manager Responsibilities Added Appendix 5.3 to clarify Item Status matrix. Made extensive updates to reflect current business practices | 08/04/08 |
| E | Reviewed document to add missing/additional content, as well as updating Roles and Responsibilities to reflect the actions currently performed as well as additions for stronger Form-Fit-Function Validation requirement. | 11/12/09 |
| F | Added to References “Product Structure Engineering Checklist for ECO and PMCR PROE1034”. Added information to EC Engineer section to comply with PROE1034. Added CORE+/ECO Process | 04/01/11 |
| G | Added ECO PM Role/Responsibility and Updated Item Status Tables in Appendix 5.3, added Appendix 5.4, defining Item Status Controls | 04/27/16 |
| H | Rewrote to reflect current Business Practices with TC, CORE+/CORE+, and Oracle | 08/28/17 |
| I | Updated to reflect SGFP TC/CORE+/SAP ECR/ECO Process | 04/23/18 |
| J | Updated to expand and define ECO PM roles and responsibilities | 10/31/2021 |
| K | Changed 03.3-03-01 to 03.3-03 | 10/11/2021 |
| L | Add Risk Analysis to DEA Review Requirements | 01/17/2025 |

APPLIED MATERIALS CONFIDENTIAL: This document and the information contained in it are confidential and cannot be copied or disclosed in whole or in part without the expressed written consent of Applied Materials.

Table of Contents

1.	Purpose and Scope	3

2.	Acronyms & Definitions	3

3.	Responsibility	6

4.	Procedure	6

5.	Appendices	13

6.	References	33

# Purpose and Scope

The purpose of this document is to describe and standardize the operating practices of the Engineering Change Request (ECR) and Engineering Change Order (ECO) Process, including individual and organizational roles and responsibilities of process participants.

In addition, this procedure will define the functional requirements for activities and interactions of all disciplines within the ECO process.

The scope of this procedure initiates from the origination of an Engineering Change Request/Engineering Change Order through the disposition, review, approval and implementation of ECOs at Applied Materials. The Roles and Responsibilities are applicable to the disciplines within all Business Units (BUs), Quality & Reliability (Q & R) and Operational units utilizing the ECO process at Applied Materials.

Policy

It is Applied Materials Corporate policy that any changes to product or product documentation be documented, approved, conveyed, and implemented by an appropriate ECO.

An ECO is a document which, along with supporting materials, defines, controls, and communicates engineering changes to affected individuals. The ECO process addresses changes in design, material, manufacturing processes, documentation and/or product structure at Applied Materials.

# Acronyms & Definitions

- AGS – Applied Global Services

- BM – Business Management

- BOM – Bill of Material

- BU – Business Unit

- CBM – Commodity Business Manager

- CCB – Change Control Board (See Triad)

- CEN – Customer Engineering News bulletins

- CIB – Change Implementation Board

- COC – Cost Of Consumables

- COO – Cost Of Ownership

- CORE – Legacy Change Order Request for Engineering Database

- CORE+ – New Version of the Change Order Request for Engineering Database

- CPO – Corporate Process Owner

- D&DC – Document and Data Control

- DCC – Document Control Center (a.k.a. Document and Data Control)

- DEA – Design Engineering Authority

- DFM – Design for Manufacturability

- E&Z – Excess and Zero

- EC – Engineering Change – as in EC Engineer or EC Manager

- ECO – Engineering Change Order

- ECO (Corrective) – ECO generated to correct drawing or design errors in response to ESW, SPS, DFM or customer issues

- ECO (Continual Improvement) – ECO generated to release either new products or options based upon new technology or enhancements to existing designs. Includes cost reductions, replacements for obsolescence, and new documents

- ECR – Engineering Change Request is a precursor to the ECO

- Engineering Change – The design and technical engineering work required to (1) Change the fit, form, or function of a part, assembly, sub-assembly, or system configuration, (2) Release or update product documentation, (3) Modify BOMs, or (4) move products through PDP phases

- ESD – Electro Static Discharge

- ESW – Engineering Specification Waiver

- FMEA – Failure Mode and Effect Analysis

- GM – General Manager

- GESCO – Global Engineering Services and Commercialization Organization

- GOME – Global Operations Manufacturing Engineering – see GESCO

- GOMO – Global Operations Manufacturing Organization

- GPM – Global Product Manager

- Inspection Point – A point where checks are performed to ensure that all required attributes of the ECO are correct and complete at that time (See ECO 9 step flow process chart). These attributes are described in the Roles and Responsibilities in Appendix 5.1

- KPI – Key Process Indicator

- MEA – Manufacturing Engineering Authority

- MLA – Master Layout Authority

- MRP – Materials Requirements Planning

- NPME – New Product Manufacturing Engineer

- NSR/CES – Non-Standard Request/Customer Engineering Special

- OEM – Original Equipment Manufacturer

- PBG – Product Business Group

- PDF – Part Definition Form

- PDM – Product Data Management system (drawing vault)

- PIK – Product Improvement Kits

- PMA – Product Management Authority

- PN – Part Number

- POA – Product Option Architecture

- Production ECO – Excludes Prototypes, Evaluation and NSR/CES ECOs

- PSE – Product Structure Engineer

- PST – Product Structure Tool

- Q & R – Quality and Reliability

- SQR – Supplier Quality and Reliability

- QMS – Quality Management System

- REA – Request for Engineering Action

- RFQ – Request for Quote

- SAP – Current ERP System used by SSG

- SDD – Software Design Document

- SPC – Statistical Process Control

- SPS – Supplier Problem Sheet

- SR – Software Request

- SRS – Software Requirements Specification

- T4S – Software that pushes TC ECO Change Object Parts/BOMs to SAP

- TC – Teamcenter Engineering PLM Database

- TPS – Total Product Support

- TQC – Touch Quality Check

- TQM – Total Quality Manager

- TRIAD – "Triad Member" refers to the following:

- Design Engineering Authority (DEA)

- Product Line Manager (PLM)

- Business Unit Operations (BUOps)

NOTE: A Change Control Board (CCB) is used to carry out the functions of the Triad

# Responsibility

- RACI Chart:

| RACI Steps | BU Engineering Managers | D&DC | EC Engineer | CCB | TQMs |
| --- | --- | --- | --- | --- | --- |
| Implementation and effectiveness of this procedure. | R/A |  |  |  |  |
| Developing, maintaining, and publishing ECO KPIs and tools on behalf of Engineering Management. |  | R/A |  |  |  |
| Integrity of the drawings, information within ECO, monitoring of the approval and implementation phases of ECO Process |  |  | A |  |  |
| Control of the changes, decision on the changes when assigned by BUs |  |  |  | R/A |  |
| Evaluation of the compliance this procedure on periodic basis |  |  |  |  | R/A |

## NOTE: ECO roles, responsibilities and disciplines are further detailed in Appendix 5.1.

# Procedure

The Corporate Change process is broken up into several different versions using the programs listed:

Concept Release (TC/SAP)

First Eval Release – aka Concept to Eval Release (TC/SAP)

Standard ECR (TC/CORE+/SAP)

Live CCB ECR (TC/CORE+/SAP)

Reduced Approval ECR (TC/CORE+/SAP)

NOTE: For the Concept Release and First Eval Release Processes, parts are revised in TC and sent into workflow.  After the TC Workflow Approvals (as applicable) are complete, the Part and BOM information is automatically pushed into SAP via T4S which automatically Implements with no user intervention.

NOTE: For the Standard, Live CCB, and Reduced Approval ECRs, ECRs are created in TC and CORE+, Approved and Scheduled in CORE+ and are automatically pushed to SAP for Implementation.

Process Governance:

ECO KPIs are reviewed on a regular basis during Operations Reviews held for each responsible area.

Each Change Type shall be reviewed for past activity at each inspection point – such responsibilities are referenced in the workflows below and are the responsibility of the assigned Reviewers/Approvers as defined via the TC, and CORE+ Databases.  Any non-compliance with policy or standards shall be recorded in TC for the Concept to Eval Workflows and in the CORE+ Issue and Comment Log for ECRs. Issues shall be resolved prior to the ECO proceeding toward approval and Engineering Management shall be informed of any continuing issue that warrants rejection of the ECO.  Concept Release workflows have no inspection points outside the workflow requirements as set in the TC workflow.

As part of Applied Materials’ comprehensive product life cycle management, ECOs are used to control change.  Examples of changes include, but are not limited to:

### Updates to Engineering Documentation via Part Revision Activities

### Create/Maintain of BOMs

### Release of New Parts and BOMs

### Changes to Item Status Values

### Changes to Critical Part Flag and Supplier Information

### Changes to Part Descriptions

Concept Release Process:

Concept to Eval Release Process:

CORE+ Standard ECR/ECO Process:

CORE+ Live CCB ECR/ECO Process:

CORE+ Reduced Approval ECR/ECO Process:

Engineering Change Process

Appendices

Appendix 5.1 – ECO Roles, Responsibilities and Disciplines

Administrative Review

The Administrative Reviewers are responsible for evaluating and managing ECO change impact by performing an Administrative Review to ensure that proposed ECOs include definitive solution statements and applicable attachments to enable the creation of new part numbers and BOMs.

The Admin Reviewer shall:

- Provide guidance to the EC Engineer during development of the ECR.

- Review the ECR to ensure all applicable elements in the ECO Checklist (03.3-02-02) are correct.

Communicate to the EC Engineer any actions required to ensure that the administrative documentation is complete. Proactively work with the EC Engineer to ensure that incomplete and incorrect forms are corrected, and that future incident are prevented.

- 	Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Business Management

The BM Representative is responsible for the following:

- Customer communication and implementation planning with forecast.

- Addressing Safety retrofits with customers and coordinating through TPS.

- Driving implementation cut-in plan through Manufacturing, collaborating with GPM.

- Providing recommendations based on business opportunities and customer requirements assessment.

Business Unit Quality & Reliability (Q & R)

Business Unit Q & R is responsible for concurrently working with the EC Engineer and others to provide and include items required by the Q & R function. BU Q & R provides information, assistance, guidance, and training to EC Engineers so that QA requirements are understood and efficiently met. BU Q & R shall review, disposition, and guide the implementation of ECOs to ensure the design is reliable and requirements are addressed within their respective BUs.

The BU Q & R function shall:

During ECR Approval:

Include all test requirements and defined product acceptance criteria.

Verify that all assembly (OMS or assembly procedures) and test instructions are included as required (PQP components only).

Identify potential changes required to QA checklists, TQC, or other existing product acceptance criteria and make appropriate modifications.

Define product MTBF and MTTR

Perform all necessary tests to verify conformance to specifications.

Validate process capabilities.

Validate AFR and NC status.

Provide guidance to the EC Engineer to ensure conformance to corporate reliability standards, if required.

Assess the impact of software / firmware changes – determine the need for additional system testing / Apps Labs / marathons.

Obtain vendor data to determine reliability for OEM components.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

AMAT Form 0250-75116  Rev. B

CIB Member

CIB Members are responsible for planning and implementing approved ECOs. The CIB is an ad hoc group representing specific functions, i.e., Planners, Buyers, Engineering, etc. determined by the nature of each ECO. Specific responsibilities for these functions are found in other documents listed in the reference section.

The CIB member shall:

During ECR Step “Enter Lead Time to Implement” (aka CIB Planning), determine the tasks to implement the change.

During implementation planning, calculate time required to perform specific functional tasks. Notify the Primary Planner of the time required by entering the number of days into CORE+.

After the effectivity date for the change has been established, implement functional tasks to meet established effectivity date. Notify the Primary Planner and EC Engineer immediately of possible schedule slippage.

Record the actual implementation date with the ECO upon completion of implementation tasks.

Notify the EC Engineer immediately if any problems arise with the approved documentation i.e., errors, and missing or incomplete information.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Cost Accounting/PBG Cost Approver

Cost Accounting is responsible for reviewing the cost impact of an Engineering Change, providing any input that will assist in proceeding with the change, and tracking the financial aspects of the Engineering Change.

Cost Accounting shall:

During CORE+ Live CCB Review

Identify those changes that have a financial impact (retrofit costs and inventory liabilities).

Validate that the BU will accept the proposed financial charges the ECO will create.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Customer Representative (a.k.a. CE! Representative)

The Customer Representative is responsible for review of the ECO documentation and qualifying data presented in ECOs to ensure conformance with customer expectations and Applied Materials established standards.

The Customer Representative shall:

During CORE+ ECR Approvals:

Communicate to external customers any impact of the change on:

- Product Maintainability

- Product Operation

- Spares

- Retrofits and PIKs.

AMAT Form 0250-75116  Rev. B

AMAT Form 0250-75116  Rev. B

Product Training

- Documentation such as product support manuals

- Special tools used in product support.

- Installation, Warranty, and contract costs.

- Safety

- Interchangeability Issues

- Software

- New hardware modifications – associated firmware changes

Communicate to EC Engineer any extraordinary data (in addition to the data provided above) that is required for the customer. In addition, recommend cut-in timeline or project to ensure appropriate customers are notified of changes prior to effective date or project. Collect data for the customer that is not originally provided by the EC Engineer and review with the customer.

- Notify the Copy Exact Manager when changing jobs to ensure that a replacement has been identified.

## NOTE: When a CE! Class A or B change is noted, 03.3-03 shall be followed.

D&DC Manager

Document and Data Control Manager shall grant or deny requests for deviation from processes detailed in this document in support of organization business needs or in resolution of conflicting requirements.

Design Engineering / Design Engineering Authority (DEA)

Design Engineering organizations are responsible for reviewing ECO content to ensure that proposed changes satisfy design intent and customer requirements and do not adversely affect the hardware and software compatibility of past, present and future system configurations.  For most groups, a single Design Engineer is designated the Design Engineering Authority for a product.  The DEA responsibilities are the same as the Design Engineers, but the DEA has other functions in addition to the Design Engineer’s responsibilities.

The Design Engineer / DEA is responsible for the product definition: including but not limited to the functionality, documentation, and Bill of Materials for the product they are assigned for the entire life of that product.

Design Engineering / DEA shall:

-	Develop new designs, including modeling/drawings in the vault.

-	Ensure all drawings to be sent for quote, review, or first article build to an external source shall have an ERP status of at least Concept.

-	Participate in their BU specific Pre-ECO process as directed by their BU management directive.

During ECR Approval:

Verify the design of the product to ensure that the impact of hardware and software changes is consistent with Applied Materials standards and the changes do not result in adverse safety issues and process reliability is not degraded. Design verification performed to support ECO changes shall be documented in accordance with Applied Materials Product Development Process.

Perform a risk analysis of replacement parts or new parts during part creation/release in Teamcenter (FMECA, if appropriate for the change) and substantiate product design with documented justification and rationale.

Involve the Corporate Safety organization in all changes affecting safety.

In conjunction with the manufacturing engineer, comply with interchangeability rules, Critical Parts rules and Copy Exact rules. Reference 03.3-10, 05.4-22, and 03.3-03.

Provide inputs to the design team with respect to the root cause to be addressed by the change and the effect of the change on the design of the piece part, component, or module.

Ensure that the Reason Code is correct. Log a Comment or Issue as required.

Include all approved and controlled Applied Materials quality requirements, which apply (i.e. cosmetic specs, workmanship standards, traceability requirements, ESD precautions, and any applicable QMS requirements).

Identify key design concerns to be addressed by ECO and ensure the ECO will not adversely affect process control.

Forward information concerning an ECO to appropriate individuals within their PBG as necessary.

Assign a Design Engineer to serve on the concurrent design team as required.

Assure that the Reason Code is correct. Log a Comment or Issue as required.

Verify Design Integrity (i.e. BOM, DfM, GD&T).

Review all documentation to ensure the documentation meets the drawing requirements as required per 03.3-02-01/0250-70606.

Validate new parts are assigned the correct commodity code per 0250-10856.

Approve ECO when requirements are satisfied.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

The Design Engineering Authority has the following additional responsibilities:

As a TRIAD and/or CCB Member, the DEA shall review, disposition, and guide the implementation of ECOs to ensure product design issues and requirements are addressed within the product group.

The DEA shall:

During CCB Review:

In conjunction with the other CCB Members, be cognizant of the status of resource loading to make accurate ECO disposition decisions.

In conjunction with the other CCB Members, review the ECO problem statement and Stakeholder inputs regarding the priority of the proposed ECO and determine if resources are available to solve the problem.

Validate the Classification of the changes as being either a Class A or Class B change for the Copy Exactly Customers, when applicable. When CE! Class A or B is noted, 03.3-03 shall be followed.

In conjunction with the other CCB Members, and inputs from product safety, shall determine and explicitly indicate if current product safety is adversely affected by the ECO or if the ECO creates potential product safety liability.

The Design Engineering Authority may:

Ensure that changes impacting critical parts include the SEMI F29 process for changes, and initiate a request for supplier evaluation and approval for new critical parts

- EC Manager

The EC Manager is responsible for the rapid and efficient processing of ECOs and management of a change process that result in cost-effective change implementation.

The EC Manager shall:

During ECR Review ensure that:

Review ECRs for completeness. Return ECRs to the EC Engineer when the problem statement is not clearly stated or the Reason Code is inappropriate.

Review BU/Product/Place and change category for the ECO.

Remove any approver when escalated appropriately per the decision of the D&DC Manager.

- The EC Manager shall also:

Assist EC Engineer and others on the change team to resolve open issues and overcome barriers to rapidly and efficiently process ECRs.

In collaboration with the other EC Managers, drive continuous improvement to the ECO process, software tools, and ECO training.

Maintain group matrices of ECO participants within CORE+ P4.

Represent the first level of escalation for process problems including reporting improper use of the policy, procedures, training program, certification, and survival guide.

- The EC Manager may:

Assist the Management in determining, obtaining, and communicating the appropriate resources to support the ECO process (including the MEA, DEA, GPM, and Stakeholders).

Assist the Management in determining and obtaining/recruiting substitute resources to support the ECO process and ensure they are effectively communicated to everyone involved in the ECO process.

EC Engineer

AMAT Form 0250-75116 Rev. B

AMAT Form 0250-75116 Rev. B

The primary role of the EC Engineer is to be the Project Manager of the change and leader of the design team that will develop the solution and supporting documentation in a cost-efficient manner that meets the needs of all functional organizations. The EC Engineer/EC Owner is CORE+ and TC certified and is responsible for the accuracy and completeness of the ECR documentation submitted. The EC Engineer will also coordinate with the primary Planner to ensure that the change is implemented effectively. The EC Engineer is ultimately responsible for all facets of a change and all impacts of his or her ECO.

The EC Engineer shall:

During ECR Creation and/or ECR Solution Formulation:

If the ECO affects a Copy Exact Customer, fill out the CE! Checklist.

Select an appropriate ECO Reason Code (ref 03.3-02-04).

In conjunction with the stakeholders, develop all documentation necessary to implement the proposed change and comply with all applicable Applied Materials design standards.

In conjunction with the Manufacturing Engineer, comply with the ECO Checklist 03.3-02-02, which includes; Interchangeability rules, Critical Parts rules and Copy Exact rules. Reference 03.3-10, 05.4-22, 03.3-03.

In conjunction with the Product Structure Engineer, comply with the Product Structure Engineering Checklist for ECO and PMCR 03.3-02-37-01.

In conjunction with the DEA, classifies the change as class A, B or "No Impact", and includes this information in the Problem/Solution statement.

Ensure that all applications, including those of other groups and platforms, of the parts are being modified are considered and addressed in the ECO.

Identify possible changes that may require changes to software and/or firmware.

Establish and convey the driving event for effectivity – Depletion of current inventory, specific machine, immediate for safety or performance issues, etc.

Gain sound material cost estimates from the materials organization for all new parts, assemblies and kits.

Identify all critical and non-critical parts in the revised item

During CIB Implementation:

Coordinate with the primary Planner to ensure that all CIB Members implement their activities.

ECO PM

AMAT Form 0250-75116 Rev. B

AMAT Form 0250-75116 Rev. B

The primary role of the ECO PM as steward of change management is to review ECRs for excellence during scope development and solution formulation to drive quality improvements and decrease ECR cycle time.

The ECO PM Shall:

- Determine if ECR category is correctly selected

- Review the ECR after Solution Formulation (from ideation to Pre-CCB) to review the scope of the change and costs involved

- Determine if the Live CCB Attribute should be Yes or No based on ECR category definitions (Live CCB for all A1 and A2, and only those B1 with cost impact >$500 (or as defined by BU), else Reduced Approvers)

  - Cost Impact defined and identified on CCB Summary tab from BU and AGS Primary Planners: Total MFG ECO + AGS Cost and ECR Specific E&Z Impact

- If Live CCB = Yes

  - Schedule and hold a Pre-CCB Meeting to review ECR scope

  - Schedule and hold a Live CCB Meeting once approved in Pre-CCB

- If Live CCB = No

  - Route ECR through Reduced Approvals

- For shared design parts, the Secondary BU ECO PM schedules and holds Live CCB Meetings in accordance to the scope of the ECR.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

ECR Creator

The ECR Creator is responsible for identifying a problem, creating an accurate and detailed ECR Problem and Solution Statement and submitting the ECR.

The ECR Creator shall:

During ECR Creation:

Provide a clear and complete Problem and Solution statement. The information provided should indicate why the problem exists and why the problem should be fixed. This information should be adequate to determine whether to approve or disapprove the ECR.

Select an appropriate ECO Reason Code.

Identify all item numbers associated with the problem. Select the most appropriate change category. Provide a descriptive title for the problem.

Determine if the EC Engineer is going to be themselves or if the ECR shall go to a Resource Focal for EC Engineer assignment.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

ECR Submitted-FYI Team

The ECR Submitted FYI Team is responsible for providing inputs to the priority of the change as well as participating in the design of the solution and the ECO.

The FYI member shall:

Review the ECR to see that it meets Stakeholder defined objectives.

Provide inputs in a timely manner to the CCB Team and resource for making priority decisions.

Participate in the design team representing Stakeholder function.

Log all issues identified within the ECR.

Resolve any issues with the EC Engineer.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Global Product Management (GPM)

The GPM, as a TRIAD and/or CCB Member, shall review, assign, disposition, guide the design, and the implementation of ECOs to ensure product management issues and requirements are addressed within their BU. Persons functioning in the GPM role are Global Product Managers or representatives from the Global Product Marketing group sanctioned by the GPM to function in this role. The PMA over the ECO shall reside with the product division GPM who has ownership of the option.

The Global Product Management Representative shall:

In conjunction with the other CCB Members, be cognizant of the status of resource loading in order to make accurate ECO disposition decisions.

In conjunction with the other CCB Members, review the ECO problem statement and Stakeholder inputs regarding the priority of the proposed ECO and if resources are available to solve the problem.

During ECR Approval:

Forward information concerning an ECO to appropriate individuals within their BU.

Participate as required in the Review and Approval portion of the concurrent design phase. Assure that the Reason Code is correct. Log a Comment or Issue as required. If a disposition is needed on the correct Reason Code the Primary GPM will make the Final Decision.

Provide inputs to the design team with respect to the scope of the change, the cost of implementing the change, the customer requirements for the change, spares and retrofit requirements as required for the change, based on the POA and strategic direction for the product (e.g. Standard, Option, etc.). In coordination with the EC Engineer, review the consumable/non-consumable part attribute and correct, as required, using the AGS flag change request process.

Develop the field communication message describing the change for the installed base and coordinate prior to implementation. Provide to EC Engineer for attachment to the ECO.  (Reference Customer Change Notification 03.3-01-03)

During CIB Plan:

As required, communicate target MRP Effectivity Date/Slot Date information to responsible parties for changes affecting ordering, configuration, and corresponding Product Management Tools. GPM shall provide reasons for setting these targets in this communication.

Notify the EC Manager when changing jobs.

Global Product Support

The Global Product Support Function is to assess potential impact of ECO identified problem and proposed solutions on the maintainability of equipment in the installed base.

The Global Product Support shall:

-  During ECR Approval:

Determine compatibility and completeness of solution as it pertains to the installed base.

AMAT Form 0250-75116  Rev. B

AMAT Form 0250-75116  Rev. B

Determine the need for, produce, and manage new retrofit or PIKs, modification of any existing PIKs, Retrofit Kits, or Preventive Maintenance Kits, and/or generation of CENs.

Communicate requirements to support the installed base to the EC Engineer.

Communicate any required designation as a CIB Member to the EC Engineer.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Manufacturing Engineering / Manufacturing Engineering Authority (MEA)

Manufacturing Engineering is responsible for reviewing Requests to assess impact on manufacturing processes and schedules. Individuals assigned to this function are required to have the technical knowledge and cognizance needed to ensure that the requirements specified below are met.  For most groups, a Manufacturing Engineer is designated as the Manufacturing Engineering Authority.  The MEA has the same responsibilities as the Manufacturing Engineer, but has additional responsibilities based on the products they are supporting.  In some cases, the MEA is also the Chief Engineer (the manager of the BU Manufacturing Engineers).

Manufacturing Engineering shall:

During ECR Approval:

Review compliance to Fit, Form and Function rules (Interchangeability), review that all Product Families affected are identified and included, ensure Copy Exact requirements are fulfilled, and ensure all associated issues are addressed and resolved.

Provide recommendations for appropriate dispositions of Revised Items.

In conjunction with the Design Engineer, comply with the ECO Checklist 03.3-02-02 that includes; Interchangeability rules, Critical Parts rules and Copy Exact rules. (Reference 03.3-10, 05.4-22, 03.3-03)

Provide inputs to the team with respect to the root cause to be addressed by the change, and the effect of the change on the manufacturing process.

Develop a solution that is appropriate with respect to manufacturing process requirements and documentation to include work instructions, test equipment, tools, fixtures, implementation instructions, rework instructions, etc. as required.

Identify key manufacturing concerns to be addressed by ECO and ensure the ECO will not adversely affect the manufacturability of the product.

Forward information concerning an ECO to appropriate individuals within their product group as necessary.

Assure that the Reason Code is correct. Log a Comment or Issue as required.

Approve, Log an Issue, or Disapprove an ECO based on its content.

AMAT Form 0250-75116  Rev. B

During CIB Plan:

Determine and/or review planned lead-time to implement, and modify as necessary based on impact to manufacturing process and schedule.

- During CIB Implementation:

Manage progress of manufacturing changes and/or adjustments. Confirm that all actions have been taken and changes made to manufacturing processes and/or tooling as required.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

The Manufacturing Engineering Authority has the following additional responsibilities:

As a member of the CCB Team, the MEA shall review; disposition and guide the implementation of ECOs to ensure manufacturing issues and requirements are addressed within his or her product group.

The MEA shall:

During CCB Review:

In conjunction with the other CCB Members, be cognizant of the status of resource loading to make accurate ECO disposition decisions.

In conjunction with the other CCB Members, review the problem statement of ECOs, and Stakeholder inputs on the priority of the proposed ECO and determine if resources are available to solve the problem.

In conjunction with the other CCB Members and input from product safety, determine and explicitly indicate if current product safety is adversely affected by the ECO or if the ECO creates potential product safety liability.

Master Layout Authority:

Master Layout Authority (MLA) shall review and implement ECOs affecting Master Layout Products as a member of the Approvers and/or CIB Team

Master Layout Authority shall:

- Ensure all parts that have a UG model should be present in the assembly in their appropriate locations.

- Validate the TC BOM matches the Model BOM within the software limitations.

- Update the Master Layout for affected product(s) at ECO Schedule per the Revised Item List.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified

Master Scheduling

Master Scheduling shall review and implement ECOs, affecting product configuration as a Member of the CIB team.

AMAT Form 0250-75116  Rev. B

Master Scheduling shall:

Update affected planning bills

Notify the EC Manager when changing jobs to ensure that a replacement has been identified

Planning / Primary Planner

Responsible for communicating the ECO impact to inventory control and production control activities, provide any input that will assist in proceeding with the change, and implement the inventory and production control aspects of the Engineering Change.

Planning / Primary Planner shall:

During the Create CDW Step:

- Create and provide an initial Cost Disposition Worksheet as required in accordance to 0250-75206.

During Solution Formulation:

Communicate impact of the change on planning activities to the EC Engineer.

AMAT Form 0250-75116  Rev. B

Evaluate all inventory levels and work with the EC Engineer to make appropriate disposition decisions and assist in completion of the cost disposition worksheet.

- During CIB Plan:

- Review all planned implementation lead-times from the CIB Members (Production ECOs only).

- During Set Effectivity:

Schedule the MRP effectivity date for the ECO based on all planned implementation lead-times, material lead-times, and inventory lead-times. Ensure that MRP effectivity date reflects project objectives determined by the GPM and/or the EC Engineer, which may minimize time and cost. Ensure that MRP effectivity date effectively minimizes rework and scrap exposure of inactive and obsolete material.

During CIB Implementation:

Update item attributes for new parts per the Primary Planner Work Instruction.

AMAT Form 0250-75116  Rev. B

During implementation, collaborate with CIB Members when implementation issues arise. When issues cannot be resolved, escalate this to the EC Manager.

If the MRP effectivity dates changes during implementation, inform everyone concerned. Keep in mind previously entered “planned lead-times to implement”.

Perform and assist the ECO Buyer with scrap and rework activities as per the directions in the ECO. Notify Buyer with the ECO disposition of the material.

Review all critical parts to ensure supplier entry onto the Approved Supplier List has been completed. Coordinate with MPM and SQR if entries are absent.

Identify and ensure transfer of new Part Numbers from default Planner code to the correct Planner codes. To identify new items, enter 30 day lead time, Buyer Name, Fixed Days Supply.

Provide final cost worksheet data.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Product Safety

Product EH&S is responsible for reviewing and approving, if acceptable, an ECO when triggered by the Safety Checklist.

During ECR Approval:

- Assure that the ECO complies with Applied Materials’ Safety Policies

Product Structure Engineer

AMAT Form 0250-75116  Rev. B

AMAT Form 0250-75116  Rev. B

Product Structure Engineering is responsible for providing the information to maintain the overall configurable product structure. This includes the review, approval, and implementation of configurable ECOs.

Product Structuring shall:

During ECR Approval:

Review the ECO contents to ensure proper structuring, and reference designators to support manufacturing and to ensure alignment with Product Option Architecture (POA)

Communicate any product structuring issues to the EC Engineer and all relevant participants

Provide guidance to the EC Engineer regarding modification to the configurable product structure to reflect changes in the requirements of the manufacturing organization per ECO specifications.

Develop or assist the EC Engineer in developing the WAS and IS reference designators for conditions to support manufacturing and ensure alignment with the product option architecture (POA).

Provide detail of the reporting structure/relationship for things such as mutually exclusive relationships or other relationships not easily represented in a reference designator.

During CIB Implementation:

Communicate the configurable product structure changes to all relevant participants

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Purchasing / ECO Buyer

The ECO Buyer is responsible for communicating and providing input to the Primary Planner that will assist him/her in setting the MRP Effectivity Date. They are responsible for communicating any information pertaining to any parts on contract to the appropriate organization. They are the primary points of contact between Applied Materials and the supplier on ECO related questions. They will act as the Central Purchasing Agent for any special engineering material requests.

The Purchasing/ECO Buyer shall:

During CIB Plan:

Establish and set lead-time and purchase cost from the identified supplier.

Identify the level of supplier inventory that Applied Materials is contractually liable for consuming, and communicate this information to the Primary Planner for Cost Disposition Worksheet input.

For long-term contract/bus-route parts, contact a CBM representative to identify the levels of supplier inventory that Applied Materials is contractually liable for consuming. Communicate this information to the Primary Planner.

AMAT Form 0250-75116  Rev. B

Provide released documentation to the identified supplier after MRP date has been established.

If conditions change and the MRP effectivity date does not allow sufficient material lead-time, may coordinate with the Primary Planner to resolve the issue (either change the effectivity date or take corrective action to ensure that the material is available within lead-time).

Act as a point of contact with the suppliers for any spot buy related activities.

Purchase new spot-buy parts when demand is loaded into SAP.

Under the request from the EC Engineer, and during the review cycle, the ECO Purchaser should contact the supplier about submitting the first article paperwork.

Perform rework and scrap activities required for inactive and obsolete items if required.

- During CIB Implementation:

Modify any open purchase orders to the new (changed) purchased parts after the effectivity date has been established if required.

Prior to implementation, establish and set lead-time and purchase cost from the identified supplier.

Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

NOTE: Some BU’s do not include Buyers as ECR FYI members. In these cases, the Primary Planner communicates changes to the Buyer for Material actions by the ECO. The Primary Planner creates an ECO summary that identifies the Buyer(s) affected by the change and the action per the ECO.

Resource Focal

The Resource Focal is responsible for assigning an appropriate engineer to analyze and solve the problem presented in the REA and work with the Engineer to ensure ample time to complete the ECO.

The Resource Focal shall:

During Solution Formulation:

Review the ECR contents.

Assign the ECR to appropriate personnel, under their direction, based on workload, availability, level of complexity and skill levels of personnel.

If workload does not permit immediate action on the Notification, contact GPM, MEA and DEA for prioritization or reassignment of ECRs.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Spares / AGS ECO Analyst

The Spares or AGS ECO Analyst is responsible for: a) Approving AGS inventory responsibility and spares support for parts moving to Eval, Production, Inactive, or Obsolete status, b) Communicating Spares inventory to be included on the cost disposition sheet when necessary, and c) Obtaining a forecast for any released parts that will have an anticipated Spares demand.

Spares Function shall:

During ECR Approval:

Review all critical parts to ensure supplier entry onto the Approved Supplier List has been completed. Coordinate with MPM or SQR if entries are absent and submit documentation for consideration to add supplier to the Approved Supplier List.

Identify any impact involving structure, customer satisfaction, and inventory to the Installed Base and ensure that the change meets all their requirements. The AGS ECO Analyst shall be an Approver on the ECO.

Assess the inventory impact of the proposed change and coordinate disposition with the EC Engineer and Manufacturing Planners.

Coordinate with the EC Engineer to ensure replacement parts are identified when applicable.

Coordinate with the EC Engineer to ensure item status is appropriately applied.

Provide guidance to the EC Engineer in developing the change to ensure viability of price targets and to resolve COO/COC impacts.

AMAT Form 0250-75116  Rev. B

Assess training and PIK impact within the Installed Base.

If inventory is impacted, report the Spares Global Inventory and pricing as necessary so that it may be included on the Cost Disposition Worksheet.

During CIB Plan:

When new parts are released and identified as spares, request a Spares Forecast from the responsible division.

Confirm data entry for cost, supplier, and critical part ASL. Log an issue until information is entered.

During CIB Implementation:

- Update SAP to reflect current change status of information.

- The Spares Representative shall serve as a CIB Member to assure that the effects of engineering changes upon spare parts inventories are properly implemented.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Technical Publications

Technical Publications is responsible for reviewing all ECRs for impact to new or existing manuals.

Technical Publications shall:

During ECR Approval:

Identify impact to new and existing manuals and incorporate necessary changes into the ECR.

- Ensure System Manuals (0230 commodity coded parts) are available.

- Notify the EC Manager when changing jobs to ensure that a replacement has been identified.

Technical Review/ D&DC Drafting

D&DC Drafting is responsible for reviewing documentation in the PDM System during initial release (to Eval Status or higher) or during revisions if requested by the part owner.  Review issues will be incorporated into drawing when applicable.

D&DC Drafting shall:

Prior to ECR or ECO submission, review documentation in Teamcenter workflow as required to

Ensure that documentation complies with Applied Materials Corporate Specifications (reference DRM, 0250-70606, and 03.3-02-01).

- OEM CAD Librarian Shall:

Review OEM parts to ensure they are in compliance with 0250-42816.

Verify Interchangeability of commercial items (OEM only).

Review critical parts to ensure compliance with 05.4-22.

Work with the EC Engineer to resolve any issues that may arise.

Test Engineering Solutions & Technology

Test Engineering Solutions & Technology (TEST) is responsible for developing and/or modifying the test process design and its documentation to ensure compliance to all TEST/Corporate Standards and qualifying data presented in an ECO. Such an ECO must either have (1) Established the need for a test fixture or (2) Initiated a change that affects an existing test fixture.

TEST performs the duties of the EC Engineer to process an ECO when either a new test fixture design is being released or an existing fixture is being modified.

Test Engineering shall:

During ECR Approval:

Determine if product design changes are being made that affect product testing.

Determine the scope and impact of the changes to test process hardware, software and/or documentation – properly structure changes for any test processes that are affected.

During CIB Plan:

Communicate to appropriate project stakeholders (e.g. Manufacturing Engineer, Production Manager, Contract Manufacturer representative, Pilot Manufacturing Program Manager, IDT Lead, etc.) the expected project timeline to help set an effectivity date.

During CIB Implementation:

- Coordinate all required modifications to maintain existing test process compliance.

- Notify the EC Manager when changing roles to ensure that a replacement has been identified.

## Appendix 5.2 – SAP Item Status Attributes and Controls

| Item Status | Issued | Concept (M0) | Prototype (M1) | Evaluation (M5) | NSR (M6) | Production (M2) |
| --- | --- | --- | --- | --- | --- | --- |
| Status Attributes | üBOM Allowed | üBOM Allowed | üBOM Allowed | üBOM Allowed | üBOM Allowed | üBOM Allowed |
|  | Build in WIP | üBuild in WIP | üBuild in WIP | üBuild in WIP | üBuild in WIP | üBuild in WIP |
|  | Customer Order Enabled | Customer Order Enabled | Customer Order Enabled | üCustomer Order Enabled | üCustomer Order Enabled | üCustomer Order Enabled |
|  | Internal Order Enabled | üInternal Order Enabled | üInternal Order Enabled | üInternal Order Enabled | üInternal Order Enabled | üInternal Order Enabled |
|  | Invoice Enabled | Invoice Enabled | Invoice Enabled | üInvoice Enabled | üInvoice Enabled | üInvoice Enabled |
|  | Transactable | üTransactable | üTransactable | üTransactable | üTransactable | üTransactable |
|  | Purchasable | üPurchasable | üPurchasable | üPurchasable | üPurchasable | üPurchasable |
|  | Stockable | Stockable | Stockable | üStockable | üStockable | üStockable |
| Other Controls | None | Expense Item | Expense Item | Invoice Asset Value | Invoice Asset Value | Invoice Asset Value |
|  |  | Document Required | Document Required | Document Required | Document Required | Document Required |
|  |  |  |  | Drawing Standard Review | Drawing Standard Review | Drawing Standard Review |
|  |  |  |  | Customer Evaluation Agreement | Approved NSR Quote |  |
| Purpose | Issued from TC - ECO Required w/Item Status Change to utilize the Item in Engineering or Production. | Used during very early product development to begin establishing Bills of Material. | For Engineering use to document and procure new designs | Used during new product development when a customer evaluation agreement exists. | Used solely by NSR organizations to release and modify NSRs following Quote Approval. | Used when full production planning and control is required |
| Available Workflow | Concept | Concept | Production | TC Evaluation | Production | Production |
|  | Eval | TC Evaluation |  | Production |  |  |
|  | Production | Production |  |  |  |  |
| AMAT PDP Phase | --undefined -- | C&F | Alpha | Late Alpha-Beta | --undefined -- | Late Beta-Gamma |

## Appendix 5.3 –Item Status Change Control Requirements

|  |  | Item Status |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EX1/ Issued (Not Released) | Concept (M0) | Proto type (M1) | Eval (M5) | NSR/ CES (M6) | Production (M2) | Inactive (G7) | Obsolete (G8/G9) |
| Shipment Controls | Shipment to Customer: Physical part | Customer agreement and General Manager Approval | Customer agreement and General Manager Approval | Enabled | Enabled | Enabled | Enabled | Enabled | AGS Spares, EPG Reconfigurations only, no new product |
|  | Shipment to Customer: Kit (unassembled, 0240, 0242..) | Customer agreement and General Manager Approval | Allowed under NSR or CAP CODE approval | Enabled | Enabled | Enabled | Enabled | Enabled | AGS Spares, EPG Reconfigurations only, no new product |
|  | Product Specification Tool (PST) Enabled | No | No | Yes- with Modeled NSR | Yes | No | Yes | Yes | N/A |
|  | Purchase Order in ERP | No | Yes | Yes | Yes | Yes | Yes | Yes | No, (AGS SAP only) |
| Change Controls | Cost Disposition on Revision | No | No | Yes | Yes | Yes | Yes | Yes | N/A |
|  | Part Number Required | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | Revision Controlled | No | Yes | Yes | Yes | Yes | Yes | Yes | Locked as "ZZ" cannot be further revised |
|  | Allowed for OEM | No | No | Legacy only | Legacy only | Legacy only | Yes | Yes | Yes |
|  | Enabled for initial release without ECO | No | Enabled | No | Yes | No | No | N/A | N/A |
|  | OMS Required | No | No | No | Yes | No | Yes | Yes | N/A |
|  | Master Layout (new product only) | No | No | No | No | No | Yes | Yes | N/A |
|  | ECR/ ECO | No | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | N/A |
|  | Engineering Specification Wavier (ESW) Allowed | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

# References

##### Applied Materials Commodity Code Definitions 0250-10856

##### Cost Disposition Worksheet Procedure 0250-75206

##### Drawing & Drafting Stds and Best Practices Manual 0250-70606

##### ECO Checklist 03.3-02-02

##### OEM Policy and Business Process in Teamcenter Engineering 0250-42816

##### Safety Checklist Procedure 11.5-03

##### Product Structure Engineering Checklist for ECO and PMCR 03.3-02-37-01

##### Purchasing Work Instruction for Production ECOs 03.3-02-22

##### Primary Planner Work Instruction 03.3-05-04

##### Technical Review Checklist 03.3-02-01